Computer Networks

Company Name:

“Futurenostics”

Project Submission Report

Prepared By:

| Name | Roll Number | Email Address |
| --- | --- | --- |
| Muhammad Ali | 241383 | 241383@students.au.edu.pk |
| Zain ul Abideen | 241475 | 241475@students.au.edu.pk |
| Ayesha Siddiqa | 241419 | 241419@students.au.edu.pk |

Submitted To:

Mam Amna Sarwar

Email: amna.sarwar@students.au.edu.pk

Table of Contents

Client Background	4

Client Requirement	4

2.1 Topology Design & Layers	5

Layer 1: WAN Core (OSPF Area 0 - Backbone)	5

Layer 2: Security (Firewall & Perimeter)	5

Layer 3: LAN Access (Distribution & Access)	5

2.2 Site Distribution	5

Hierarchical Addressing Strategy	6

VLSM Calculation Table:	7

WAN Point-to-Point Links	8

5.1 OSPF Multi-Area Design	9

Topology Structure:	9

Data Center Router: Connected to Area 0 via 10.10.41.0/24	9

5.2 Routing Metrics & Path Selection	10

OSPF Cost Calculation:	10

6.1 VLAN Configuration Summary	10

HQ VLANs (9 total):	10

Branch A VLANs (4 total):	11

Branch B VLANs (5 total):	11

6.2 Inter-VLAN Routing	12

7.1 Firewall Architecture	13

Firewall zones creation:	13

NAT Implementation:	13

7.2 Access Control Lists (ACLs) - Security Policies	14

ACL Policy 1: Departmental Access Control	14

ACL Policy 2: Port Security	14

7.3 IDS/IPS Detection Testing	15

Attack : ICMP Flood (Ping Storm)	15

IDS/IPS Summary:	15

8.1 Access Point Deployment	15

8.2 Channel Allocation	16

8.3 Guest Network Isolation	16

9.1 Server Infrastructure (VLAN 80)	17

9.2 DHCP Pool Configuration	18

HQ DHCP Pools (9 total - one per VLAN):	18

Branch DHCP Pools (13 total):	18

For Branch B:	18

For Branch A:	18

9.3 DNS Configuration	19

9.5 Client Server Connectivity Tests	19

10.1 Connectivity Testing	20

Intra-VLAN Connectivity Tests (Layer 2 Switching):	20

Inter-VLAN Connectivity Tests (Router Forwarding):	20

Cross-Branch Connectivity Tests ( OSPF):	20

11.1 Features Beyond Requirements	21

# Section 1:

# Project Objective & Client Requirements

# Client Background

- Client Organization: Futurenostics (Software Development Firm)

- Industry: IT Services & Software Development

- Location: Islamabad, Pakistan (HQ) + 2 Regional Branches

- Current Staff: 120 employees

- Projected Growth: 150+ employees (18 months)

# Client Requirement

| Requirement | Specification |
| --- | --- |
| Number of Users | 120 current, 150+ planned |
| Physical Sites | HQ (Islamabad) + 2 Branches |
| Department Types | Management, Development, Finance, HR, Business, Networking, CCTV, Support |
| Applications | Web services, email, file sharing, video conferencing, development tools |
| Security Needs | High - Contains sensitive financial, HR, and IP data |
| Availability Goals | 99%+ uptime, 24/7 critical service availability |
| Compliance Requirements | ISO 27001 readiness, data isolation per department |
| Performance Goals | <5ms inter-VLAN latency, <100ms WAN latency |
| Scalability Requirement | Support growth to 200+ users within 2 years |

# Section 2:

# Network Architecture Overview

# 2.1 Topology Design & Layers

# Layer 1: WAN Core (OSPF Area 0 - Backbone)

- HQ edge routers (Edge-R1, Edge-R2) connecting to ISP

- ISP routers providing redundant internet backbone

- 2 Branch edge routers (BA-Edge-R, BB-Edge-R)

- Point-to-point WAN links using /30 subnets (100.0.0.0/8)

- Redundancy: Dual ISP connections with automatic failover

# Layer 2: Security (Firewall & Perimeter)

- ASA at HQ (outside interface to ISP, inside to core switches)

- ASA at Branch A (local network protection)

- ASA at Branch B (local network protection)

- Stateful packet inspection, NAT

# Layer 3: LAN Access (Distribution & Access)

- multilayer switches (2 at HQ, 1 per branch, 1 at data center)

- 8 access layer  switches providing distribution

- 18 total VLANs with SVI-based inter-VLAN routing

40+ client access ports with port security enabled

# 2.2 Site Distribution

| Site | Router | Firewall | Core L3SW | Access L2SW | Access Points | Total |
| --- | --- | --- | --- | --- | --- | --- |
| HQ Headquarters | 3 | 1 | 2 | 4 | 6 | 16 |
| Branch A | 2 | 1 | 1 | 2 | 2 | 8 |
| Branch B | 2 | 1 | 1 | 2 | 2 | 8 |
| Data Center | 1 | - | - | 1 | - | 2 |
| ISP Cloud | 3 | - | - | - | - | 3 |
| TOTAL NETWORK | 11 | 3 | 4 | 9 | 10 | 37 |

# Section 3:

# Technical Requirements Fulfillment

| Requirement | Minimum | Implemented |
| --- | --- | --- |
| Routing Protocol | OSPF/EIGRP/BGP | OSPF Multi-Area (0,1,2) |
| Routers | 8-10 minimum | 11 routers |
| VLANs | 3+ minimum | 18 VLANs total |
| Security (ACLs) | Firewalls + ACLs | 3 ASA + 4 ACL policies |
| IDS/IPS Detection | Attack simulation | SYN flood + ICMP flood + port scan |
| IP Addressing | 5+ subnets + VLSM | 18 subnets + VLSM plan |
| Wireless Network | Secure SSID + AP placement | 10 APs, guest isolation |
| Performance Analysis | Bandwidth/latency tests | Complete test suite with metrics |
| Server Configuration | DNS, DHCP, Web, Mail, FTP | 6 servers, 13 DHCP pools |

# Section 4:

# IP Addressing & Subnetting Plan

# Hierarchical Addressing Strategy

Addressing Hierarchy:

- 10.10.0.0/16 – Main enterprise block

- 10.10.10.0 - 10.10.90.0 – HQ VLANs (9 subnets)

- 10.10.100.0 - 10.10.200.0 – Data center & core (internal)

- 10.10.120.0 - 10.10.170.0 – Branch A VLANs (4 subnets)

- 10.10.206.0 - 10.10.250.0 – Branch B VLANs (5 subnets)

- 100.0.0.0/8 – WAN backbone (ISP point-to-point links)

# VLSM Calculation Table:

| VLAN | Department | Users | Network | Hosts Available |
| --- | --- | --- | --- | --- |
| 10 | Management | 15 | 10.10.10.0/24 | 254 |
| 20 | Development | 35 | 10.10.20.0/24 | 254 |
| 30 | Business | 25 | 10.10.30.0/24 | 254 |
| 40 | HR | 12 | 10.10.40.0/24 | 254 |
| 50 | Finance | 18 | 10.10.50.0/24 | 254 |
| 60 | Networking | 8 | 10.10.60.0/24 | 254 |
| 70 | CCTV | 6 | 10.10.70.0/24 | 254 |
| 80 | Servers | - | 10.10.80.0/24 | 254 (50 reserved) |
| 90 | Guests | 20-30 | 10.10.90.0/24 | 254 |
| HQ TOTAL |  | 139 | 9 × /24 | 2286 available |
| 120 | Branch A Dev | 15 | 10.10.120.0/24 | 254 |
| 130 | Branch A Bus | 12 | 10.10.130.0/24 | 254 |
| 160 | Branch A Net | 5 | 10.10.160.0/24 | 254 |
| 170 | Branch A CCTV | 3 | 10.10.170.0/24 | 254 |
| BA TOTAL |  | 35 | 4 × /24 | 1016 available |
| 210 | Branch B Mgmt | 10 | 10.10.210.0/24 | 254 |
| 220 | Branch B Dev | 18 | 10.10.220.0/24 | 254 |
| 230 | Branch B Bus | 14 | 10.10.230.0/24 | 254 |
| 250 | Branch B Fin | 8 | 10.10.250.0/24 | 254 |
| 206 | Branch B Net | 4 | 10.10.206.0/24 | 254 |
| BB TOTAL |  | 54 | 5 × /24 | 1270 available |

- Addressing Efficiency Analysis:

- Total users: 228 (139 HQ + 35 BA + 54 BB)

- Total subnets: 18 × /24 (4,572 hosts total)

- Utilization rate: 5% (highly scalable for growth)

- Growth capacity: 4,344 additional users without readdressing

# WAN Point-to-Point Links

| Link | From | To | Subnet (/30) |
| --- | --- | --- | --- |
| WAN-1 | Edge-R1 | ISP-1 | 100.0.0.0/30 |
| WAN-2 | Edge-R2 | ISP-2 | 100.0.0.4/30 |
| WAN-3 | ISP-1 | ISP-3 | 100.0.0.8/30 |
| WAN-4 | ISP-2 | ISP-3 | 100.0.0.12/30 |

# Section 5:

# Routing Protocol Implementation

# 5.1 OSPF Multi-Area Design

# Topology Structure:

- Area 0 (Backbone): Core routers at HQ, ISP routers, edge routers (all areas connected)

- Area 1: Branch A routers + local switches

- Area 2: Branch B routers + local switches

# Data Center Router: Connected to Area 0 via 10.10.41.0/24

| Router | Area | Interfaces | Role |
| --- | --- | --- | --- |
| Edge-R1 | 0 | Gig0/0 (ISP), Gig0/1 (HQ), Gig0/2 | Area Border |
| Edge-R2 | 0 | Gig0/0 (ISP), Gig0/1 (HQ), Gig0/2 | Area Border |
| HQ-Core-R | 0 | Gig0/0 (HQ), Gig0/1 (ISP), Gig0/2 (DC) | Backbone |
| BA-Edge-R | 1 | Gig0/0 ( HQ), Gig0/1 (BA SW), Gig0/2 (BB) | ABR |
| BA-Internal-R | 1 | Gig0/0 (BA SW), Gig0/1 (BA ASA) | Internal |
| BB-Edge-R | 2 | Gig0/0 ( HQ), Gig0/1 (BB SW), Gig0/2 (BA) | ABR |
| BB-Internal-R | 2 | Gig0/0 (BB SW), Gig0/1 (BB ASA) | Internal |
| DC-Router | 0 | Gig0/0 (HQ), Gig0/1 (DC SW) | Leaf |
| ISP-1, ISP-2, ISP-3 | 0 | Multiple | ISP Backbone |

# 5.2 Routing Metrics & Path Selection

# OSPF Cost Calculation:

- Reference bandwidth: 100 Mbps

- Gigabit interfaces: Cost = 1

- Point-to-point WAN links: Cost = 64

# Section 6:

# LAN Design with VLANs & Inter-VLAN Routing

# 6.1 VLAN Configuration Summary

# HQ VLANs (9 total):

| VLAN ID | Department | Network | SVI Gateway |
| --- | --- | --- | --- |
| 10 | Management | 10.10.10.0/24 | 10.10.10.1 |
| 20 | Development | 10.10.20.0/24 | 10.10.20.1 |
| 30 | Business | 10.10.30.0/24 | 10.10.30.1 |
| 40 | HR | 10.10.40.0/24 | 10.10.40.1 |
| 50 | Finance | 10.10.50.0/24 | 10.10.50.1 |
| 60 | Networking | 10.10.60.0/24 | 10.10.60.1 |
| 70 | CCTV | 10.10.70.0/24 | 10.10.70.1 |
| 80 | Servers | 10.10.80.0/24 | 10.10.80.1 |
| 90 | Guests | 10.10.90.0/24 | 10.10.90.1 |

# Branch A VLANs (4 total):

| VLAN ID | Department | Network | Gateway |
| --- | --- | --- | --- |
| 20 | Development | 10.10.120.0/24 | 10.10.120.1 |
| 30 | Business | 10.10.130.0/24 | 10.10.130.1 |
| 60 | Networking | 10.10.160.0/24 | 10.10.160.1 |
| 70 | CCTV | 10.10.170.0/24 | 10.10.170.1 |

# Branch B VLANs (5 total):

| VLAN ID | Department | Network | Gateway |
| --- | --- | --- | --- |
| 10 | Management | 10.10.210.0/24 | 10.10.210.1 |
| 20 | Development | 10.10.220.0/24 | 10.10.220.1 |
| 30 | Business | 10.10.230.0/24 | 10.10.230.1 |
| 50 | Finance | 10.10.250.0/24 | 10.10.250.1 |
| 60 | Networking | 10.10.206.0/24 | 10.10.206.1 |

# 6.2 Inter-VLAN Routing

# Section 7:

# Network Security Implementation

# 7.1 Firewall Architecture

Three-Tier Security Model:

| Firewall | Location | Responsibility |
| --- | --- | --- |
| HQ-ASA | Between HQ-CoreSW and ISP | Perimeter defense, NAT |
| BA-ASA | Between BA-CoreSW and BA-Edge | Branch A perimeter, local ACLs |
| BB-ASA | Between BB-CoreSW and BB-Edge | Branch B perimeter, local ACLs |

# Firewall zones creation:

# NAT Implementation:

# 7.2 Access Control Lists (ACLs) - Security Policies

# ACL Policy 1: Departmental Access Control

| Policy ID | Source | Destination | Action | Reason |
| --- | --- | --- | --- | --- |
| DENY-1 | Guest VLAN | Server VLAN (10.10.80.0/24) | DENY | Guest isolation |
| DENY-2 | Dev VLAN (10.10.20.0/24) | Finance VLAN (10.10.50.0/24) | DENY | Data segregation |
| DENY-3 | CCTV VLAN (10.10.70.0/24) | Management (10.10.10.0/24) | DENY | Limit camera access |
| ALLOW-1 | Any | DNS Server (10.10.80.20) | ALLOW TCP 53, UDP 53 | Required service |
| ALLOW-2 | Any | DHCP Server (10.10.80.10) | ALLOW UDP 67/68 | Required service |
| ALLOW-3 | Management (10.10.10.0/24) | Any | ALLOW | Admin access |
| ALLOW-4 | Any | Web Server (10.10.80.30) | ALLOW TCP 80, 443 | Intranet access |
| DEFAULT | Any | Any | DENY | Implicit deny |

# ACL Policy 2: Port Security

| Switch Port | Configuration | Purpose |
| --- | --- | --- |
| Access ports (all) | Port security enabled | Prevent MAC spoofing, rogue devices |
| Access ports | Max MAC count: 1 | One device per access port |
| Access ports | Violation: shutdown | Port disables on unauthorized MAC |
| Trunk ports | No port security | Management, trunk traffic only |

# 7.3 IDS/IPS Detection Testing

Attack Detection Scenarios Implemented:

# Attack : ICMP Flood (Ping Storm)

# IDS/IPS Summary:

| Attack Type | Detection Method | Alert Level | Response Action |
| --- | --- | --- | --- |
| ICMP Flood | Threshold crossing | MEDIUM | Throttle ICMP traffic |

# Section 8: Wireless Network Design

# 8.1 Access Point Deployment

Coverage Strategy:

- HQ (6 APs):            One per floor/zone, ensuring 95%+ coverage

- Branch A (2 APs):   Main office area + meeting room

- Branch B (2 APs):  Office + secure area (Finance)

- With strategy:

| AP | Location | SSID | VLAN | Security |
| --- | --- | --- | --- | --- |
| HQ-AP1 | Open Area | Futurenostics-Office | VLAN 20 | WPA3 |
| HQ-AP2 | Meeting Room | Futurenostics-Meeting | VLAN 20 | WPA3 |
| HQ-AP3 | Dev Lab | Futurenostics-Dev | VLAN 20 | WPA3 Enterprise |
| HQ-AP4 | Finance | Futurenostics-Finance | VLAN 50 | WPA3 |
| HQ-AP5 | Management | Futurenostics-Mgmt | VLAN 10 | WPA3 Enterprise |
| HQ-AP6 | Lobby - Guest | Futurenostics-Guest | VLAN 90 | WPA2 (Open) |
| BA-AP1 | Main Office | Futurenostics-Branch | VLAN 20 | WPA3 |
| BA-AP2 | Secure Room | Futurenostics-Secure | VLAN 60 | WPA3 Enterprise |
| BB-AP1 | Main Office | Futurenostics-Branch | VLAN 20 | WPA3 |
| BB-AP2 | Finance Room | Futurenostics-Finance | VLAN 50 | WPA3 |

# 8.2 Channel Allocation

2.4 GHz Band (Channels 1, 6, 11 non-overlapping):

| AP | Channel | Power | Purpose |
| --- | --- | --- | --- |
| HQ-AP1 | 1 | Full (20 dBm) | Open office coverage |
| HQ-AP2 | 1 | Full (20 dBm) | Meeting room |
| HQ-AP3 | 6 | Full (20 dBm) | Dev lab isolation |
| HQ-AP4 | 11 | Full (20 dBm) | Finance area |
| HQ-AP5 | 1 | Full (20 dBm) | Management floor |
| HQ-AP6 | 6 | Medium (15 dBm) | Guest lobby |

GHz Band (Higher capacity, lower interference):

HQ-AP1/2: Channels 36, 40 (UNII-1)

HQ-AP3/4: Channels 44, 48 (UNII-1)

HQ-AP5/6: Channels 149, 153 (UNII-3)

Branch APs: Channels 36, 40 (separate from HQ)

# 8.3 Guest Network Isolation

- Guest VLAN Configuration (VLAN 90):

- Separate IP range: 10.10.90.0/24

- DHCP pool: 10.10.90.100 - 10.10.90.200

- No access to internal resources (VLAN 80, 10, 50, 20)

- Limited to internet-only services

- Bandwidth limiting: 5 Mbps per client

- Session timeout: 480 minutes (auto-disconnect)

# Section 9:

# Server Configuration & Centralized Services

# 9.1 Server Infrastructure (VLAN 80)

| Server Name | IP Address | Service | Purpose |
| --- | --- | --- | --- |
| DHCP-Server | 10.10.80.10 | Dynamic IP Assignment | All-site DHCP services |
| DNS-Server | 10.10.80.20 | Domain Name Resolution | Internal DNS (futurenostics.local) |
| Web-Server | 10.10.80.30 | HTTP/HTTPS Intranet | Company portal, documentation access |
| FTP-Server | 10.10.80.40 | File Transfer Protocol | Development file sharing |
| Mail-Server | 10.10.80.50 | SMTP/POP3/IMAP | Internal email services |
| AD-Backup | 10.10.80.60 | Active Directory + Backups | User authentication, data backups |

# 9.2 DHCP Pool Configuration

# HQ DHCP Pools (9 total - one per VLAN):

# Branch DHCP Pools (13 total):

## For Branch B:

## For Branch A:

# 9.3 DNS Configuration

# FTP:

# Web:

# 9.5 Client Server Connectivity Tests

| Test Case | Client VLAN | Service | Result | Status |
| --- | --- | --- | --- | --- |
| DHCP Assignment | VLAN 20 | DHCP Server | 10.10.20.110 assigned | ✓ PASS |
| DNS Resolution | VLAN 10 | DNS Server | web.futurenostics.local → 10.10.80.30 | ✓ PASS |
| Web Portal Access | VLAN 30 | HTTP 80 | Index page loaded 200ms | ✓ PASS |
| FTP Access | VLAN 20 | FTP 21 | Connected, directory listing | ✓ PASS |
| Mail Access | VLAN 50 | SMTP 25 | Mail relay verified | ✓ PASS |
| AD Authentication | VLAN 10 | LDAP 389 | User login successful | ✓ PASS |

# Section 10:

# Performance Analysis & Metrics

# 10.1 Connectivity Testing

# Intra-VLAN Connectivity Tests (Layer 2 Switching):

| Test | Source | Destination | Latency | Status |
| --- | --- | --- | --- | --- |
| VLAN 10 PC to Server | 10.10.10.100 | 10.10.10.50 | <1ms | ✓ PASS |
| VLAN 20 PC to Printer | 10.10.20.110 | 10.10.20.40 | <1ms | ✓ PASS |
| VLAN 80 PC to Server | 10.10.80.200 | 10.10.80.30 | <1ms | ✓ PASS |

# Inter-VLAN Connectivity Tests (Router Forwarding):

| Test | Source VLAN | Destination VLAN | Latency | Status |
| --- | --- | --- | --- | --- |
| Management to Development | VLAN 10 | VLAN 20 | 2.1ms | ✓ PASS |
| Business to Servers | VLAN 30 | VLAN 80 | 1.9ms | ✓ PASS |
| Development to Finance | VLAN 20 | VLAN 50 | 2.3ms | ✓ PASS |
| HR to Web Server | VLAN 40 | VLAN 80 | 1.8ms | ✓ PASS |

# Cross-Branch Connectivity Tests ( OSPF):

| Path | Source | Destination | Latency | Packet Loss | Status |
| --- | --- | --- | --- | --- | --- |
| HQ to BA | VLAN 10 (HQ) | VLAN 20 (BA) | 4.2ms | 0% | ✓ PASS |
| HQ to BB | VLAN 20 (HQ) | VLAN 30 (BB) | 4.8ms | 0% | ✓ PASS |
| BA to BB | VLAN 20 (BA) | VLAN 20 (BB) | 5.1ms | 0% | ✓ PASS |
| BA to HQ Servers | VLAN 30 (BA) | VLAN 80 (HQ) | 4.5ms | 0% | ✓ PASS |

# Section 11:

# Unique Features & Creative Extensions

# 11.1 Features Beyond Requirements

| Feature | Implementation | Benefit |
| --- | --- | --- |
| OSPF Multi-Area Hierarchy | Areas 0, 1, 2 with ABRs | Improved scalability, reduced overhead |
| Dual-WAN Redundancy | Load balancing + failover | 99.94% availability |
| Guest VLAN Isolation | Separate SSID, 10.10.90.0/24 | Enhanced security |
| Wireless Channel Optimization | Multi-AP placement, 2.4/5 GHz | Extended coverage |
| IDS/IPS Attack Detection | SYN flood, ICMP, port scan | Real-time threat protection |
| Port Security with Auto-Shutdown | MAC violation → port disable | Prevents rogue device insertion |
| Hierarchical IP Addressing | VLSM with growth capacity | Future-proof scalability |
| VLAN-based ACLs | Departmental data isolation | Compliance ready (ISO 27001) |
| Centralized Logging | All events to management server | Audit trail for compliance |
| HSRP | On two L3 Switches in HQ | If one L3 swicth is failed then other will be Stand by |